LED 200 module light beam light

instructions

Please keep the instructions properly

1. Remove the packaging

For proper and safe use, please read the instructions carefully before installing and using this product. This specification contains important installation and application information. Please strictly follow the operation procedures when installing and operating the product. At the same time, please keep this instruction manual properly.

The 200 module beam lamp is designed and produced in strict accordance with CE standards, in line with the international standard DMX512 signal protocol, can be used alone, can also be used online, with fast rotation, low noise, powerful features, suitable for small and medium-sized concerts, theaters, studios, nightclubs and bars and other places.

Please remove the package carefully, check whether the product is damaged during transportation, and check whether the following contents are complete.

200 module beam lamp- - - - 1                signal line- - - -1

Manual- - -1                                             This power cable- - - -1

- 2 Safety instructions

pay attention to!

Please be careful before operating this product, this product is high voltage product, if contact wire may be electric shock!

This product is a good product before delivery. In order to keep the product in good condition and ensure safe operation, users should follow the safety matters and the instructions.

 Important: Damage caused by not following this instruction is not covered by the warranty. The supplier is not responsible for the product problems arising from this.
 If the product has been exposed to extreme unstable temperature conditions (e. g., after transportation), do not supply the product immediately because water drops due to temperature changes may damage the product. Please use the product has returned to normal temperature.

This product can be used in the 90-240V voltage range and is intended for indoor use. Please ensure that the ground voltage is not higher than the tolerable range of the product!! The power plug must be plugged into a protective class I socket. Green or tea-cyan conductors must be grounded.

Please check the product-level power cord frequently. Ensure that the power cord is not folded or damaged and not connected to other wires! Special attention is required when connecting the power cord or the related wiring. Unplug the power supply without using the product or before cleaning.

Before using the product, please be familiar with the operation function of the product. Please do not expose children or no professionals to the product.

Please do not shake this product. Do not use brute force when installing or operating the product. Do not allow unprofessionals to operate the product. Most of the damage is caused by an unprofessional operation.

The product is not equipped with relevant repair accessories. Product maintenance and repair can only be carried out by professionals!

Please do not change this product without authorization, otherwise the product may be damaged, so the damage caused is not covered by the warranty. Also, unprofessional operations can lead to short circuits, burns or electric shocks, and so on.

3. Operating instructions

- -This product is a head dyeing beam lamp used to create dyeing performance.

- -If the product after a large temperature difference (for example, after transportation), do not immediately start the product, because the heat account cold shrinkage will damage the product. Please wait until the device reaches the normal room temperature before starting the product.

- -Pay attention to the shock shock. Avoid strong collisions during product installation.

- -Please do not raise the whole product through the lamp head, because the mechanical nature of the equipment may be damaged.

- -Make sure the product is not exposed to overheated, wet, or dusty places. Please do not place any wires on the ground or you may be in danger of electric shock.

- -Before installing the product, please ensure the installation point of safety

- -Please attach the product to the safety rope and check that all the screws are correct during installation

- -Please make sure that the lens is in good condition. Replace the lens when the lens is damaged or scratched.

- -It is recommended that the technicians familiar with the product operate the product. Non-technical personnel are prohibited from operating this equipment because many losses are the result of non-professional operations.

- -Please keep the packing materials properly for secondary transportation as required.

- -Do not change the product without the guidance from the manufacturer or distributor.

- -Any equipment failure caused by failing to use the equipment is not covered by the warranty. And any accident caused by short circuit, injury, electric shock, ultraviolet light caused by injury, bulb explosion accidents are not within the scope of warranty.

. 4. Product hanging and installation

Note: For more safety, install this product away from the walkway, seating area, or accessible area

Before hanging the product, ensure that the installation point bears 10 times the weight of the product.

Product installation must have a double protection device, such as a safety rope.

When hanging, removing or repairing the product, it is forbidden to stand under the installation point.

Please ensure that this product should be installed at least 0.5 meters away from the flammable materials.

Please remember to use the safety rope as a safety guarantee in case the accident is dropped quickly.

Hanging point: the top hanging requires that the installation personnel must be experienced, including calculating the load-bearing requirements, the installation materials used, and the periodic inspection of the safety status of the installation materials and products. If you lack this knowledge, please do not try to install it yourself. If not incorrectly, it may lead to serious consequences such as personal injury.

Make sure that all the required hanging and installation steps are completed before the product is powered on.

Quick lock hanging: there are specially designed professional pendants at the bottom of this product, including quick lock pendant and safety rope hanging points (please refer to the figure below).

When hanging this product to the truss, please remember to use a suitable quick lock, fix it to the quick lock pendant position of the product, and hang it with the screws of M10. To further ensure the safety, please install the safety rope to the safety rope hanging point at the bottom of the product, and fix it to the truss.

Whatever hanging you choose, please remember to use a safety rope. The bottom of the product is equipped with a special safety rope hanging point, please refer to the above figure, and please remember to use the specially provided safety rope hanging point, do not put the safety rope in the handle position.

6.DMX-512 linkage

Please plug the male header of the XLR signal line into the signal output port of the controller and the master head into the signal input port on the back board of the product. You can talk about multiple products in series, the signal line needs to be three-heart, shielded, with XLR input and output head. Please refer to the figure below.

DMX-512 connection with terminal (loop connection)

During installation, a DMX terminal (loop connection) is recommended if the signal line is laid over a long distance, or in noisy electrical environments, such as dance halls. This avoids the digital control signal opacity due to electrical noise. The DMX terminal consists of a simple XLR connector with 2 cores and 3 cores connecting a 120 ohm resistance. Please insert the terminal into the XLR output port of the last product, please refer to the following figure.

. 7 Menu

| primary menu | Sublevel menu | Three-level menu / parameters |
| --- | --- | --- |
| System Settings | running mode | DMX / voice control / self walk 1 / self walk 2 |
|  | channel pattern | 21CH |
|  | Horizontal reversal | Open / close |
|  | Vertical reversal | Open / close |
|  | Hall error correction | Open / close |
|  | Optical coupling error correction | Open / close |
|  | The signal to keep | Open / close |
|  | Screen protection | Open / close |
|  | Screen flip | Open / close |
|  | Synchronous update | Open / close (This function should remove DMX offline) |
|  | language | centre /EN |
|  | factory data reset |  |
| manual mode | Manual control of the channel values | 000-255 |
| system calibration | enter password | Lighting calibration |
| system reset | Effect motor reset | Effect disc reset other than XY |
|  | Scan the motor for reset | The XY axis was reset only |
|  | Full motor reset | Lamps reset |
| system info | Reset information | Display the specific information and content |
|  | And DMX data detection | Display the specific DMX detection content |
|  | sensor information | Display the optical coupling, Hall specific information |
|  | Hardware version | Displays the hardware version number |
|  | software release | Displays the software version number |

◆ System Settings

| option | explain |  |
| --- | --- | --- |
| running mode | The lamps support three operating modes: DMX mode, self-travel mode and voice control mode |  |
|  | DMX pattern | The console mode, receive DMX signal, and RDM signal |
|  | Self-walking mode 1 | The lamps are run automatically according to the built-in program 1 |
|  | Self-walking mode 2 | The lamps are run automatically according to the built-in program 2 |
|  | Sound control mode | When the lamp detects a strong sound, the lamp automatically runs a scene according to the built-in program, otherwise maintain the last scene |
| channel pattern | The following channel values can be selected: 21 CH |  |
| Horizontal reversal | Set the X-axis rotation direction |  |
|  | close | Don't reverse |
|  | open | opposite direction |
| Vertical reversal | Set the Y-axis rotation direction |  |
|  | close | Don't reverse |
|  | open | opposite direction |
| Hall error correction | Check whether the color plate, focus adjustment and other motors are reset and out of step |  |
|  | close | Position is not corrected after a misstep |
|  | open | Position is automatically corrected after loss of step |
| Optical coupling error correction | XY was tested for loss of step and corrected |  |
|  | close | Position is not corrected after a misstep |
|  | open | Position is automatically corrected after loss of step |
| The signal to keep | Output status of the lamp with no DMX signal |  |
|  | close | There is no signal, so the motor and the light source return to the position and state when the reset is complete |
|  | open | No signal, keeping the last frame of the DMX data output |
| Screen protection | The screen is bright for a long time |  |
|  | close | Often bright |
|  | open | In a static environment, the screen timing off the screen |
| Screen flip | Set the display direction of the screen |  |
|  | close | No reversal is shown |
|  | open | Reverse display |
| Synchronous update | Synchronize the setting parameters or calibration parameters of multiple lamps |  |
|  | close | The synchronous update function is turned off. |
|  | open | Connect multiple lamps with DMX cable and update the calibration interface.(Note: Remove the DMX signal wire connected to the console) |
| language | Chinese and English menu switch |  |
|  | centre | the Chinese language |
|  | EN | English menu |
| factory data reset | Lighting parameters are returned to the factory settings |  |
|  | cancel | res sic stantibus |
|  | affirm | Light ture to factory settings |

◆ manual control

This interface is used to control the current lamp, as detailed in the channel table.

Press OK to enter the edit state. Press Up and Down to change the channel value. Press OK again to save the modified value, exit editing, and press Exit not to save the modified value, and directly exit editing.

◆ system calibration

Here to set a layer of password to prevent non-professionals from misoperation. Press OK to verify the password.

| option | explain |
| --- | --- |
| Initial position calibration | After entering the sub-interface, the reset position of the motor such as X axis, Y axis, color disk, solid map, atomization, and focusing can be adjusted to make up for the error on the hardware installation. The adjustment range of 0~255,127 means that there is no adjustment. |
| Travel calibration | After entering the sub-interface, the stroke of X-axis, Y-axis, atomization, colorful and focusing motors can be adjusted. The adjustment range of 0~255,127 means that there is no adjustment. |
| calibration of power | The maximum power of the adjustable whole lamp, and the adjustment range of 0~255,255 means no adjustment. |
| MIC sensitivity calibration | Adjust the sound-control sensitivity. |
| change password | Set the system calibration password. |

◆ reset

Press Up and Down to switch the reset mode, and press OK to reset directly.

| option | explain |
| --- | --- |
| Effect motor reset | Color disc, focusing and other motor reset |
| Scan the motor for reset | The XY axis was reset only |
| Full motor reset | Lamps reset |

◆ system info

| option | explain |
| --- | --- |
| Reset the error message | If the red ERR indicator is illuminated, the lamp is running wrong: 1) IC1 communication fault (communication failure between motor and display board) 2) X / Y optical coupling error reporting 3) Color plate, focusing and other motor reset failed |
| And DMX data monitoring | From this, you can enter the subinterface to display the channel values for viewing and turn the page |
| sensor information | 1) Hall: X X X X - sensor: - X and Y optical coupling: XXXX- -XXXX |
| Hardware version number | Lighting hardware information . XX XX .XX Display board version. Motor version |
| Software version number | Lamp software version . XX XX .XX Display board version. Motor version |

. 8 Channel table

| 21CH | The channel name | The channel value | Channel function |
| --- | --- | --- | --- |
| 1 | X axle | 0-255 | 0-540 degrees |
| 2 | X axis fine-tuning | 0-255 | 0-2 Degrees |
| 3 | Y axle | 0-255 | 0-270 degrees |
| 4 | Y axis fine-tuning | 0-255 | 0-1 degrees |
| 5 | XY velocity | 0-255 | From fast to slow |
| 6 | stroboflash | 0-3 | opening the light |
|  |  | 4-99 | Synchronous flash |
|  |  | 100-149 | Pulse flash |
|  |  | 150-199 | Flash |
|  |  | 200-249 | Random flash |
|  |  | 250-255 | light |
| 7 | aiming | 0-255 | With 0-100% dimming |
| 8 | colour disk | 0-4 | white |
|  |  | 5-9 | Color 1 |
|  |  | 10-14 | Color 2 |
|  |  | 15-19 | Color 3 |
|  |  | 20-24 | Color 4 |
|  |  | 25-29 | Color 5 |
|  |  | 30-34 | Color 6 |
|  |  | 35-39 | Color 7 |
|  |  | 40-44 | Color 8 |
|  |  | 45-49 | Color 9 |
|  |  | 50-54 | Color 10 |
|  |  | 55-59 | Color 11 |
|  |  | 60-64 | Color 12 |
|  |  | 65-69 | Color 13 |
|  |  | 70-74 | White + color 1 |
|  |  | 75-79 | Color # 1 + color # 2 |
|  |  | 80-84 | Color # 2 + color # # 3 |
|  |  | 85-89 | Color # 3 + color # # 4 |
|  |  | 90-94 | Color # 4 + color # # 5 |
|  |  | 95-99 | Color # 5 + color # # 6 |
|  |  | 100-104 | Color # 6 + color # 7 |
|  |  | 105-109 | Color # 7 + color # # 8 |
|  |  | 110-114 | Color # 8 + color # # 9 |
|  |  | 115-119 | Colour 9 + color 10 |
|  |  | 120-124 | Colour 10 + color 11 |
|  |  | 125-129 | Colour 11 + color 12 |
|  |  | 130-134 | Colour 12 + color 13 |
|  |  | 135-139 | Color is 13 + white |
|  |  | 140-195 | From fast to slow counterclockwise flow water |
|  |  | 196-199 | Stop running water |
|  |  | 200-255 | From slow to fast clockwise flowing water |
| 9 | Solid plate | 0-7 | White aperture |
|  |  | 8-14 | Pattern 1 |
|  |  | 15-21 | Pattern 2 |
|  |  | 22-28 | Pattern 3 |
|  |  | 29-35 | Pattern 4 |
|  |  | 36-42 | Pattern 5 |
|  |  | 43-49 | Pattern 6 |
|  |  | 50-56 | Pattern 7 |
|  |  | 57-63 | Pattern 8 |
|  |  | 64-70 | Pattern 9 |
|  |  | 71-77 | Pattern 10 |
|  |  | 78-84 | Pattern 11 |
|  |  | 85-91 | Pattern 12 |
|  |  | 92-98 | Pattern 13 |
|  |  | 99-105 | Pattern 1 shakes from slow to fast |
|  |  | 106-112 | Pattern 2 shakes from slow to fast |
|  |  | 113-119 | Pattern 3 shakes from slow to fast |
|  |  | 120-126 | Pattern 4 shakes from slow to fast |
|  |  | 127-133 | Pattern 5 shakes from slow to fast |
|  |  | 134-140 | Pattern 6 shakes from slow to fast |
|  |  | 141-147 | Pattern 7 shakes from slow to fast |
|  |  | 148-154 | Pattern 8 shakes from slow to fast |
|  |  | 155-161 | Pattern 9 shakes from slow to fast |
|  |  | 162-168 | The tern 10 from slow to fast |
|  |  | 169-175 | Pattern 11 shakes from slow to fast |
|  |  | 176-182 | Pattern 12 shakes from slow to fast |
|  |  | 183-189 | Pattern 13 shakes from slow to fast |
|  |  | 190-221 | From fast to slow counterclockwise flow water |
|  |  | 222-223 | Stop running water |
|  |  | 224-255 | From slow to fast clockwise flowing water |
| 10 | Colorful atomization | 0-127 | Colorful atomization cut out |
|  |  | 128-192 | Fog cut in |
|  |  | 193-255 | Colorful cut in |
| 11 | prism | 0-31 | The prism cut out |
|  |  | 32-63 | Prism cut in |
|  |  | 64-255 | The prism rotates from slow to fast |
| 12 | focus | 0-255 | From far to near |
| 13 | Light band flash | 0-4 | opening the light |
|  |  | 5-250 | Synchronous flash |
|  |  | 251-255 | opening the light |
| 14 | Light with dimming | 0-255 | Dimming (from dark to bright) |
| 15 | The light with red | 0-255 | Red (from dark to bright) |
| 16 | The light with green | 0-255 | Green (from dark to bright) |
| 17 | The light is blue | 0-255 | Blue (from dark to bright) |
| 18 | Built-in color | 0-255 | Prospect color gradient |
| 19 | Dynamic effect | 0-4 | of no avail |
|  |  | 5-9 | Effect 1 |
|  |  | 10-14 | Effect 2 |
|  |  | 15-19 | Effect 3 |
|  |  | 20-24 | Effect 4 |
|  |  | 25-29 | Effect 5 |
|  |  | 30-34 | Effect 6 |
|  |  | 35-39 | Effect 7 |
|  |  | 40-44 | Effect 8 |
|  |  | 45-49 | Effect 9 |
|  |  | 50-54 | Effect 10 |
|  |  | 55-59 | Effect 11 |
|  |  | 60-64 | Effect 12 |
|  |  | 65-69 | Effect 13 |
|  |  | 70-74 | Effect 14 |
|  |  | 75-79 | Effect 15 |
|  |  | 80-84 | Effect 16 |
|  |  | 85-89 | Effect 17 |
|  |  | 90-94 | Effect 18 |
|  |  | 95-99 | Effect 19 |
|  |  | 100-104 | Effect 20 |
|  |  | 105-109 | Effect 21 |
|  |  | 110-114 | Effect 22 |
|  |  | 115-119 | Effect 23 |
|  |  | 120-124 | Effect 24 |
|  |  | 125-129 | Effect 25 |
|  |  | 130-134 | Effect 26 |
|  |  | 135-139 | Effect 27 |
|  |  | 140-144 | Effect 28 |
|  |  | 145-149 | Effect 29 |
|  |  | 150-154 | Effect 30 |
|  |  | 155-159 | Effect 31 |
|  |  | 160-164 | Effect 32 |
|  |  | 165-169 | Effect 33 |
|  |  | 170-174 | Effect 34 |
|  |  | 175-179 | Effect 35 |
|  |  | 180-184 | Effect 36 |
|  |  | 185-189 | Effect 37 |
|  |  | 190-194 | Effect 38 |
|  |  | 195-199 | Effect 39 |
|  |  | 200-204 | Effect 40 |
|  |  | 205-209 | Effect 41 |
|  |  | 210-214 | Effect 42 |

|  |  | 215-219 | Effect 43 |
| --- | --- | --- | --- |
|  |  | 220-224 | Effect 44 |
|  |  | 225-229 | Effect 45 |
|  |  | 230-234 | Effect 46 |
|  |  | 235-239 | Effect 47 |
|  |  | 240-244 | Effect 48 |
|  |  | 245-249 | Effect 49 |
|  |  | 250-254 | Effect 50 |
|  |  | 255 | Effect 51 |
| 20 | Effect speed | 0-127 | Forward from slow to fast |
|  |  | 128-255 | Reverse from slow to fast |
| 21 | reset | 230-235 | Head motor reset over 5 seconds |
|  |  | 240-245 | The XY motor is reset over 5 seconds |
|  |  | 250-255 | All times are reset over 5 seconds |

9. Product maintenance and cleaning

Please note the following points when testing this product:

- All the screws and parts of the installed products must be tightly connected and do not rust.

- The shell, lens, fixture and installation site (e. g., ceiling, hanger, truss, etc.) cannot be deformed.

- Mechanical moving parts should not have wear and tear, can not rotate unbalanced.

- The power cord should not have any damage, material broken skin or scar, etc.

The installation and use of the product must be carried out by trained professional and technical personnel, and must ensure that the safety risks are eliminated.

Note: Please unplug the power cord before performing product maintenance.

In order to maintain the product in good condition and extend the service life, we recommend routine cleaning of the product.

1. Clean the external lens and internal lens of the product every week to prevent the light output of the product from weakening due to the dust of the lens.

2. Clean the fan every week.

3. Every three months, professional electrical engineers will check the product in a detailed circuit, to ensure that the circuit contact is good, to prevent the circuit contact from aging due to overheating.

We recommend that the products be cleaned regularly. Please be careful not to use damp or linen, or never use alcohol or solvents.

This product is not equipped with replacement parts, please refer to the "Installation Instructions" section.

If users need any spare parts, please purchase them from any local dealer.