- 18 six section 4-in-1 Dyeing handkerchief lamp DMX channel menu

- AH80 DH281

8-channel mode (A001 status)

| channel | function | explain |
| --- | --- | --- |
| CH1 | Total dimming | Total dimming, from dark to bright |
| CH2 | R dimming | Red dimming, from dark to bright |
| CH3 | G dimming | Green dimming, from dark to bright |
| CH4 | B dimming | Blue dimming, from dark to bright |
| CH5 | W dimming | White dimming, from dark to bright |
| CH6 | Total stroboscopic | Global stroboscopic, from slow to fast |
| CH6 | Function selection | 0: dimming mode; |
|  |  | 1 － 50: color selection (selected by ch8); |
|  |  | 51-82: single jump effect 1-single jump effect 32; |
|  |  | 83-100: jump all effects; |
|  |  | 101-132: single gradient effect 1-single gradient effect 32; |
|  |  | 133-150: gradient all effects; |
|  |  | 151-182: single pulse effect 1-single pulse effect 32; |
|  |  | 183-200 pulse change all effects; |
|  |  | 201-255: voice control (ch8 selects voice control 1 and 2) |
| CH8 | Function speed | Function speed, from slow to fast (color selection for color output and voice control for stroboscopic and jump) |

28 channel mode (D001)

| channel | function | explain |
| --- | --- | --- |
| CH1 | Total dimming | Total dimming, from dark to bright |
| CH2 | Total stroboscopic | Global stroboscopic, from slow to fast |
| CH3 | Function selection | 0: dimming mode; |
|  |  | 1 － 50: color selection (selected by CH4); |
|  |  | 51-82: single jump effect 1-single jump effect 32; |
|  |  | 83-100: jump all effects; |
|  |  | 101-132: single gradient effect 1-single gradient effect 32; |
|  |  | 133-150: gradient all effects; |
|  |  | 151-182: single pulse effect 1-single pulse effect 32; |
|  |  | 183-200 pulse change all effects; |
|  |  | 201-255: voice control (CH4 selects voice control 1 and 2) |
| CH4 | Function speed | Function speed, from slow to fast (color selection for color output and voice control for stroboscopic and jump) |
| CH5 | R1 dimming | Red 1 dimming, from dark to bright |
| CH6 | R2 dimming | Red 2 dimming, from dark to bright |
| CH7 | R3 dimming | Red 3 dimming, from dark to bright |
| CH8 | R4 dimming | Red 4 dimming, from dark to bright |
| CH9 | R5 dimming | Red 5 dimming, from dark to bright |
| CH10 | R6 dimming | Red 6 dimming, from dark to bright |
| CH11 | G1 dimming | Green 1 dimming, from dark to bright |
| CH12 | G2 dimming | Green 2 dimming, from dark to light |
| CH13 | G3 dimming | Green 3 dimming, from dark to bright |
| CH14 | G4 dimming | Green 4 dimming, from dark to bright |
| CH15 | G5 dimming | Green 5 dimming, from dark to bright |
| CH16 | G6 dimming | Green 6 dimming, from dark to light |
| CH17 | B1 dimming | Blue 1 dimming, from dark to bright |
| CH18 | B2 dimming | Blue 2 dimming, from dark to bright |
| CH19 | B3 dimming | Blue 3 dimming, from dark to bright |
| CH20 | B4 dimming | Blue 4 dimming, from dark to bright |
| CH21 | B5 dimming | Blue 5 dimming, from dark to bright |
| CH22 | B6 dimming | Blue 6 dimming, from dark to bright |
| CH23 | W1 dimming | White 1 dimming, from dark to bright |
| CH24 | W2 dimming | White 2 dimming, from dark to bright |
| CH25 | W3 dimming | White 3 dimming, from dark to bright |
| CH26 | W4 dimming | White 4 dimming, from dark to bright |
| CH27 | W5 dimming | White 5 dimming, from dark to bright |
| CH28 | W6 dimming | White 6 dimming, from dark to bright |

- 2、 Description of nixie tube display menu

| Serial number | content | numerical value | Function and description |
| --- | --- | --- | --- |
| 1 | A001 | 001- 512 | DMX512 address code setting -8-channel mode |
| 2 | d001 | 001- 512 | DMX512 address code setting -28 channel mode |
| 3 | r255 | 000 - 255 | Red dimming, from dark to bright. |
| 4 | G255 | 000 - 255 | Green dimming, from dark to bright. |
| 5 | b255 | 000 - 255 | Blue dimming, from dark to bright. |
| 6 | u255 | 000 - 255 | White dimming, from dark to bright. |
| 7 | FH99 | 00 - 99 | No stroboscopic at 00, global stroboscopic at 01-99, from slow to fast. |
| 8 | CL01 | 01 - 49 | Manual color selection mode. |
| 9 | CC99 | 01 - 99 | Jump from slow to fast. |
| 10 | DE99 | 01 - 99 | Gradient, from slow to fast. |
| 11 | CP99 | 01 - 99 | Pulse changes from slow to fast. |
| 12 | Auto |  | Automatic effects. |
| 13 | SU01 | 01 - 02 | Mode: 1 refers to stroboscopic jump, 2 refers to jump |

- Master slave description

- In D001 or A001 status, the slave signal is automatically received without additional setting.